周向阳

吴明峰

周 园

2007-10-22

# DD DFB922512H

CL291

# SPECIFICATION FOR APPROVAL

TO:   CL291                         SAMPLE ISSUE NO.: YL0710054

MODEL: DFB922512H 防水功能(IP55)         SAMPLE ISSUE DATE: 2007-10-22

PART NAME:  DC FAN(RoHS)          SAMPLE Q, TY: 5PCS

REV: 0

- SCOPE:

THIS SPECIFICATION DEFINES THE ELECTRICAL AND MECHANICAL

### CHARACTERISTICS OF THE DC BRUSHLESS FAN.THE FAN

MOTOR IS WITH TWO PHASES AND FOUR POLES.

- CHARACTERS:

ITEM                                   DESCRIPTION

RATED VOLTAGE                    12 VDC

## OPERATION VOLTAGE             10.8~13.2VDC

STARTING VOLTAGE               9VDC(ON/OFF)

INPUT CURRENT                     0.23±10﹪A

SPEED                               2600 RPM+10%

(10 MINUTE/ ROOM TEMPERATURE 25℃)

AIR FLOW

(AT ZERO STATIC PRESSURE)               52.85CFM

AIR PRESSURE

(AT ZERO AIRFLOW)                      2.93mmH2O

ACOUSTICAL NOISE (AVG.)             31.69dB-A

SAFETY TYPE                        UL  TUV  CE  CSA

(Continued)

Page: 1

#### YOUNG LIN TECH CO., LTD,

#### MODEL: DFB922512H

INSULATION STRENGTH               10 MEG OHM MIN.AT 500 VDC

(BETWEEN FRAME AND (+) TERMINAL)

DIELECTRIC STRENGTH               5 mA  MAX. AT 500 VAC 60 HZ

ONE MINUTE, (BETWEEN FRAME AND

(+) TERMINAL)

LIFE EXPECTANCE              50,000 HOURS CONTINUOUS OPERATION

AT 25℃ WITH 15~65 %RH.

##### ROTATION                          WIDDERSHINS VIEW

FROM FRAME PLATE SIDE

OVER CURRENT SHUT DOWN       INAPPLICABILITY

LEAD WIRE                        UL 1007– AWG #24

BLACK BLACK : (-)

RED RED :(+)

###### NOTES: 1. ALL READINGS ARE MEASURED AFTER STABLY WARMING UP

THROUGH 10 MINUTES.

2. THE VALUES WRITTEN IN PARENS, ARE LIMITED SPEC.

3. ACOUSTICAL NOISE MEASURING CONDITION:

DC FAN

MICROPHONE

AIR FLOW

1M

NOISE IS MEASURED AT RATED VOLTAGE IN FREE AIR IN ANECHOIC

CHAMBER WITH B & K SOUND LEVEL METER WITH MICROPHONE AT

A DISTANCE OF ONE METER FROM THE FAN INTAKE

Page: 2

YOUNG LIN TECH CO., LTD,

MODEL: DFB922512H

- MECHANICAL:

3-1. DIMENSIONS                              SEE DIMENSIONS DRAWING

3-2. FRAME                                             PLASTIC UL: 94V-0

3-3. IMPELLER                                          PLASTIC UL: 94V-0

3-4. BEARING SYSTEM                                 TWO BALL BEARING

3-5. WEIGHT                                                 84 GRAMS

- ENVIRONMENTAL:

4-1. OPERATLNG TEMPERATURE                     -10 TO + 70 DEGREE C

4.2. STORAGE TEMPERATURE                       -40 TO + 75 DEGREE C

4-3. OPERATING HUMIDITY                                    5 TO 90 % RH

4-4. STORAGE HUMIDITY                                      5 TO 95% RH

- PROTECTION:

5-1. DROP TEST

IN MINIMUM PACKAGING CONDITION FAN WITHSTANDS EACH ONE

DROP OF THREE FACES FROM 30 CM DISTANCE HEIGHT ONTO 10 MM

THICKNESS OF WOODEN BOARD.

5-2. LOCKED ROTOR PROTECTION

IMPEDANCE OF MOTOR WINDING PROTECTS MOTOR FROM FIRE IN 15

MINUTE OF LOCKED ROTOR CONDITION AT THE RATED VOLTAGE.

- RE OZONE DEPLETING SUBSTANCES;

6-1.RESTRICTION SUBSTANCES OF SAMPLE ARE AS FOLLOWS:

Pb(PPM)＜1000         Cd(PPM)＜100        Hg(PPM)＜1000

Cr6+ (PPM)＜1000       PBBs (PPM)＜1000    PBDEs (PPM)＜1000

| # Y YLTC | Research & Development Department |  |  |
| --- | --- | --- | --- |
|  | Approved BY: 周向阳 | Checked BY: 吴明峰 | Prepared BY: 周园 |

Page: 3

IMPORTANT NOTES & GENERAL INSTRUCTIONS

- Any item which is needed to add into specification shall be determined on customer’s prior written request. If no information given , fan will be delivered based on our standard judgment.

- Customer shall confirm the matching and reliability of fan on actual set or unit application.

This include confirmation on set or unit life, electrical noise, mechanical noise,

Vibration static electricity ,electric power noise ,drift ,electric resonance between

motor and control circuit ,mechanical resonance between motor and chassis,

irregular movement of set due to motor noise ,irregular movement of set in

strong electromagnetic field ,damaged by lightning surge earthing method etc

- In order to improve the performance within the scope of specification, parts or material changes are subject to prior notice to customer.

- Any revisions on the specification shall be done based on mutual discussion and agreement.

- When any trouble occurs, both parties shall discuss on this specification to solve the matters. In this case, our guarantee is only limited to fans

Page: 4